Interface

1、gConnReader

Prototype: BOOL gConnReader(char *port, int addr=0)

Function : Connect reader

Paramters: port->Serial port number, example "com1"

addr->Set reader address here, this parameter is appoint by

programer, the parameter range is 0~9

Return: Return true when successful，else return false

2、gDiscReader

Prototype: BOOL gDiscReader(int addr=0)

Function : Disconnect reader

Paramters: addr->Same as gConnReader

Return: Return true when successful，else return false

3、ATA_StandWrite（ATA_ProteWrite）

Prototype: ATA_StandWrite(BYTE page, BYTE block, BYTE lock,

BYTE *block_data, int addr=0)

ATA_ProteWrite(BYTE page, BYTE block, BYTE lock,

BYTE *block_data, BYTE* password, int addr=0)

Function : Write card, use ATA_ProteWrite if the card has password, else use ATA_StandWrite

Paramters: page->page address，this card has 2 page, start from 0,

page 1 is UID, it can't be write by user

block->block address, range from 0 to 7,

block 0 is configuration block

lock->0x00 = unlock block

0x01 = lock block

block_data->input block data，4 BYTES

password->password of card, the data of block 7

addr->Same as gConnReader

Return: communication right return true, else return false,

call ATA_ReadCardRF32 receive data to check if it write OK

when return true

4、ATA_Access（ATA_AccessPWD）

Prototype: ATA_Access(BYTE page, BYTE block, int addr=0)

ATA_AccessPWD(BYTE page, BYTE block, BYTE* password, int addr=0)

Function : Read block data, call ATA_AccessPWD if the card has password,

else call ATA_Access

Paramters: page->Same as ATA_StandWrite

block-> Same as ATA_StandWrite

password-> Same as ATA_StandWrite

addr->Same as gConnReader

Return: communication right return true, else return false,

call ATA_ReadCardRF32 to receive data

5、ATA_ReadCardRF32

Prototype: ATA_ReadCardRF32 (BYTE *data_card, int *data_len, int addr=0)

Function : Receive card data

Paramters: data_card->return card data

data_len ->return card data length

addr->Same as gConnReader

Return: Success return 1, false return 2, communication error return 3